Switch 0:

en

config t

vlan 10

name ICT

exit

vlan 20

name CSE

exit

interface FastEthernet 3/1

switchport mode trunk

exit

interface FastEthernet 0/1

switchport mode access

switchport access vlan 10

exit

interface FastEthernet 1/1

switchport mode access

switchport access vlan 20

exit

Switch 1:

en

config t

vlan 10

name ICT

exit

vlan 20

name CSE

exit

interface FastEthernet 3/1

switchport mode trunk

exit

interface FastEthernet 2/1

switchport mode access

switchport access vlan 10

exit

interface FastEthernet 1/1

switchport access vlan 20

exit

Switch 1 to Router

interface FastEthernet 0/1

switchport mode trunk

exit

Router (Router 1):

en

config t

interface GigabitEthernet 0/0

no shutdown

exit

interface GigabitEthernet 0/0.10

encapsulation dot1Q 10

ip address 192.168.1.254 255.255.255.0

no shutdown

exit

interface GigabitEthernet 0/0.20

encapsulation dot1Q 20

ip address 192.168.2.254 255.255.255.0

no shutdown

exit